Protocol Sensitivity in Unsupervised Industrial Anomaly Detection: Threshold Transfer, Retraining Risk, and Inference Cost

Sarjan Shah1 and Abhi Shah1

1Department of Information Technology, Lok Jagruti University, Vadodara, Gujarat, India

shahsarjan968@gmail.com

abhi.shah@ljku.edu.in

Abstract

Comparisons of unsupervised anomaly detectors for industrial sensor data often report a winning method, but the winner can depend on evaluation choices that are rarely varied. This paper shows how three such choices change the conclusions: which data define the normal training period (a widely shared re-upload of SWaT places the attack period’s normal rows in the file named for normal operation), how detection thresholds are calibrated, and whether retraining data are selected with labels. Nine lightweight detectors and a small LSTM autoencoder are evaluated under standard protocols with threshold-free metrics (AUROC, AUPRC), threshold-dependent metrics under two calibration rules, event recall and block-bootstrap confidence intervals on three benchmarks: SKAB (pump and valve testbed), SWaT (water treatment) and HAI (turbine, boiler and water-treatment hardware-in-the-loop testbed). On SWaT’s standard split, all ten detectors rank attacks equally well (AUROC 0.83–0.86, overlapping intervals); on SKAB with pooled training, none is reliably better than chance, while SKAB’s per-run protocol yields run-level AUROC up to 0.79 and an inverted ranking; on HAI, the ranking differs again (PCA reconstruction and LOF lead with AUROC 0.93 and 0.92, Elliptic Envelope trails at 0.66). Protocol choices moved MCC by up to 0.8 for the same detector. With thresholds set only from normal data, the z-score, One-Class SVM and Isolation Forest were the most dependable on SWaT (MCC 0.64–0.72), while LOF, PCA reconstruction and the autoencoder raised alarms on about 70% of normal readings in the attack week; on HAI the same label-free rule held false alarms at 0.4–1.6% for most detectors, so threshold failure is dataset-specific. Only the z-score and One-Class SVM were dependable on both (MCC 0.60–0.72). In a streaming simulation, SWaT’s normal operation drifted enough to raise a static Isolation Forest’s false-alarm rate from 12% to 76%; retraining restored detection only when retraining data were known to be normal; a drift trigger saved some refits relative to a schedule but its checks cost more than refitting every hour; and label-free retraining failed. Single-reading cost in a one-CPU, 1 GB container ranged from microseconds to about 700 ms. The results argue for choosing detectors by threshold stability and retraining needs and for reporting protocols in enough detail to tell which of these a benchmark number measures.

Keywords: unsupervised anomaly detection; multivariate time series; evaluation protocol; concept drift; retraining policy; resource-constrained inference

Sarjan Shah et al. This is an open-access article distributed under the terms of the Creative Commons Attribution 3.0 United States License, which permits unrestricted use, distribution, and reproduction in any medium, provided the original author and source are credited.

Introduction

Pumps, valves and treatment plants are instrumented with sensors whose joint behaviour changes before or during a fault. Because faults and attacks are rare and costly to label, condition monitoring often relies on unsupervised (novelty) detectors that are fitted to normal operation and flag departures from it (Chandola et al., 2009; Mukherjee, 2021). In practice, false alarms decide whether such a monitor is trusted, which is why deployed systems add contextual filtering and adaptive thresholds on top of the detector (Lee, 2017). For an engineer choosing such a detector for a resource-limited monitoring node, three practical questions matter: which detector separates anomalies from normal operation best, whether that answer survives the passage of time once the detector is deployed, and what the detector costs to run.

Published comparisons give less guidance on these questions than their headline numbers suggest. Large benchmark studies report that no family of methods is consistently best and that deep models often do not beat well-chosen classical ones (Audibert et al., 2022; Schmidl et al., 2022; Sarfraz et al., 2024). A parallel line of work shows that evaluation choices, such as point adjustment, metrics that reward trivial detectors, and flawed benchmark labels, can manufacture apparent progress (Kim et al., 2022; Garg et al., 2021; Wu & Keogh, 2023; Liu & Paparrizos, 2024). These critiques concern mainly metrics and labels. Less attention has been paid to the other protocol choices an applied study must make: how the training and test data are defined, how a detection threshold is calibrated, and, once deployed, how and on which data a detector is retrained.

This study began as a conventional detector comparison on the Skoltech Anomaly Benchmark (SKAB) (Katser & Kozitsin, 2020) and the Secure Water Treatment testbed (SWaT) (Goh et al., 2016; Mathur & Tippenhauer, 2016). An initial analysis, built with choices that are common in applied work, named clear winners on each dataset and a large benefit from drift-triggered retraining. A subsequent audit showed that those conclusions rested on three protocol choices: the SWaT copy used (a public re-upload) stores the normal-labelled rows of the attack period in the file named for normal operation, so the model had been trained on the normal intervals between attacks rather than on the separate normal run; thresholds had been calibrated with different rules for different detectors; and retraining data had been selected with ground-truth labels. Instead of only correcting these choices, this paper measures how much each one changes the conclusions, and adds a third benchmark to test whether the corrected findings hold.

The contributions are empirical and methodological, not algorithmic:

- A controlled comparison of nine lightweight unsupervised detectors and a small LSTM autoencoder on SKAB, SWaT and HAI under standard, documented protocols, reported with threshold-free metrics (AUROC, AUPRC), threshold-dependent metrics (MCC, event recall, false-alarm rate) and block-bootstrap confidence intervals (Section 5).

- A protocol-sensitivity analysis showing how the split definition, the training protocol and the threshold rule change the measured performance and the ranking of detectors (Section 5.4).

- A retraining-policy study on SWaT’s genuine normal run and attack period that compares never, periodic, drift-triggered and continuous retraining, with retraining data selected with or without labels (Section 6).

- Single-reading latency, throughput, model size and peak memory for every detector inside a one-CPU, 1 GB ARM64 container (Section 7).

The study answers four research questions:

RQ1

Under standard, documented protocols with uncertainty estimates, do lightweight detectors differ in how well they rank anomalies on SKAB, SWaT and HAI?

RQ2

How much do the split definition, the training protocol and the threshold rule change measured performance and the ranking of detectors?

RQ3

Does drift-triggered retraining outperform periodic or continuous retraining, and what happens when retraining data are selected without labels?

RQ4

What does each detector cost per reading under a one-CPU, 1 GB memory cap?

Related Work

Detectors.

Classical unsupervised detectors differ in what they treat as normal. Distance- and density-based methods such as the Local Outlier Factor (LOF) score a point by the relative density of its neighbourhood (Breunig et al., 2000); boundary methods such as the One-Class SVM enclose the bulk of the training data (Schölkopf et al., 2001); covariance methods flag large Mahalanobis distances, with the Minimum Covariance Determinant (MCD) giving a robust estimate (Rousseeuw & Van Driessen, 1999); PCA reconstruction scores the residual outside a low-rank normal subspace (Shyu et al., 2003); Isolation Forest isolates points by random partitioning (Liu et al., 2008); and the histogram- and copula-based HBOS, COPOD and ECOD combine per-feature tail probabilities and are fast and nearly parameter-free (Goldstein & Uchida, 2016; Li et al., 2023; Zhao et al., 2019). Deep sequence models, including LSTM encoder-decoders (Malhotra et al., 2016), prediction-error detectors (Hundman et al., 2018), adversarial autoencoders (Audibert et al., 2020), graph networks (Deng & Hooi, 2021) and transformers (Tuli et al., 2022), report strong results on SWaT, usually with protocols and metrics that differ from paper to paper.

What benchmark studies agree on.

Comparisons that put conventional, machine-learning and deep detectors side by side find that no family wins consistently (Audibert et al., 2022), that multivariate problems are harder on average (Schmidl et al., 2022), and that simple or linear baselines match many recent deep models once inflated metrics are removed (Sarfraz et al., 2024). Tabular benchmarks reach similar conclusions (Goldstein & Uchida, 2016; Han et al., 2022). A result of the form “detector X wins on dataset A but not on B” is therefore expected and is not, on its own, a contribution.

Where evaluations go wrong.

Point adjustment credits a whole event when any of its points is flagged and can make random scores look competitive (Kim et al., 2022). Point-wise F1 and accuracy can reward detectors that flag almost everything, which the Matthews correlation coefficient (MCC) does not (Chicco & Jurman, 2020; Garg et al., 2021). Threshold-dependent metrics make comparisons hinge on the threshold rule; threshold-free AUC measures and their range-aware extensions avoid this (Paparrizos et al., 2022; Liu & Paparrizos, 2024), and range-based precision and recall account for events rather than points (Tatbul et al., 2018). Benchmark data can itself be flawed (Wu & Keogh, 2023), and the PHM community has called for shared measurement and evaluation practice for health-monitoring methods (Weiss & Brundage, 2021). These studies examine metrics and labels; the protocol choices examined here (split semantics, threshold calibration rule, and the label assumption in retraining) are complementary.

Drift and retraining.

Concept-drift research distinguishes detecting a change, understanding it and adapting to it (Gama et al., 2014; Lu et al., 2019). Two-sample tests on feature distributions are a simple and competitive way to detect shift (Rabanser et al., 2019); adaptive windows such as ADWIN resize the reference automatically (Bifet & Gavaldà, 2007); streaming anomaly benchmarks such as NAB score detectors online (Ahmad et al., 2017). For an anomaly detector, adapting to drift carries a specific risk: data used for retraining may contain the very anomalies the detector should flag. Retraining policies have been compared directly for anomaly detectors in IT operations, where periodic (blind) retraining performed slightly better than retraining triggered by a drift detector and the best retraining window depended on the detector (Poenaru-Olaru et al., 2024); a recent preprint on streaming classifiers likewise reports periodic retraining outperforming error- and ADWIN-triggered policies except under recurring drift (Dasari, 2026). Those studies retrain on data whose labels or normality are known. The present study asks the same question for lightweight detectors on industrial-control data and adds the factor a deployed detector cannot avoid: whether the retraining data are selected with labels, by the detector’s own predictions, or not at all.

Resource cost.

MLPerf Tiny measures accuracy, latency and energy of single inferences on constrained hardware, including an anomaly-detection task (Banbury et al., 2021). Comparable cost reporting for classical multivariate detectors on industrial data is rare. Here, cost is measured as single-reading latency, batch throughput, model size, load time and peak memory under explicit CPU and memory caps. Energy is not measured.

Datasets

SKAB.

SKAB was recorded on a water-circulation testbed (pump, pipes, valves) with eight sensors sampled at 1 Hz: two accelerometers (RMS), motor current, pressure, water temperature, pump-housing temperature, voltage and volumetric flow rate (Katser & Kozitsin, 2020). It contains one 9,401-row anomaly-free run and 34 fault runs (46,860 rows in total) in three families: valve 1 (16 runs), valve 2 (4) and other faults (14). Faults are physically induced and labelled per second. Each file is an independent experiment.

SWaT.

SWaT is a six-stage water-treatment plant with 51 sensors and actuators sampled at 1 Hz (Goh et al., 2016; Mathur & Tippenhauer, 2016). The standard release has a seven-day normal run (22–28 December 2015, 496,800 rows) and an attack period (28 December–2 January, 449,919 rows) in which 54,621 rows (12.1%) are labelled as attacks. The copy used in this study is a public re-upload (Kaggle, vishala28/swat-dataset-secure-water-treatment-system, v3) in which (i) normal.csv contains the normal run twice and, in its first 395,298 rows, the normal-labelled rows of the attack period, and (ii) attack.csv contains only the 54,621 attack rows. In this copy seven channels are blank throughout the normal run (MV101, AIT201, MV201, P201, P202, P204, MV303). The standard split was reconstructed by de-duplicating the normal run by timestamp and merging the attack-period rows of both files in time order, which reproduces the published row counts. The 24 continuous sensors that are present in both periods are used (AIT201, a continuous analyser, is excluded because it is blank in the normal run). Following common practice, the first 21,600 rows of the normal run (start-up stabilisation) are discarded (Li et al., 2019). Results should be re-checked on the official iTrust release.

HAI.

HAI (version 21.03) comes from a hardware-in-the-loop testbed in which a steam-turbine process, a boiler process and a water-treatment process are coupled through a simulator, sampled at 1 Hz (Shin et al., 2020; Shin et al., 2021). It provides three normal training files (921,603 rows) and five test files (402,005 rows) in which 2.2% of rows belong to 50 scripted attacks. Of its 78 points, the 55 with more than ten distinct values in the training files are treated as continuous and used. HAI adds a second process type beyond water and a test set with a much lower anomaly rate than SWaT.

Method

Features

For each continuous sensor, the raw value, 5-second and 30-second causal rolling means, a 5-second rolling standard deviation and the first difference are computed (40 features on SKAB, 120 on SWaT, 275 on HAI). Features are computed on each complete recording before any rows are removed, so windows never join readings across a removed fault and, at test time, use only past readings. The initial analysis’ elapsed-time feature was removed: its range differs between training (up to 9,960 s on SKAB) and test segments (at most 291 s), and it has no meaning for a monitor that runs continuously. Its effect is reported in Section 5.4. Wall-clock features remain excluded because they leak the identity of the recording session.

Detectors

Nine classical detectors are evaluated, all producing a continuous anomaly score: a maximum absolute z-score; PCA reconstruction error (95% of variance); Elliptic Envelope (robust MCD on ten whitened principal components, needed because the features are collinear); Isolation Forest (100 trees); One-Class SVM (RBF kernel, ν=0.1); LOF (k=20, novelty mode); and ECOD, COPOD and HBOS from PyOD (Zhao et al., 2019), with library defaults. On SWaT and HAI, One-Class SVM and LOF are fitted on a 20,000-row evenly spaced subsample of the training data because their cost grows quadratically; on HAI, ECOD and COPOD are fitted on a 100,000-row subsample because PyOD rescores test data jointly with the stored training data and the full set exceeded 16 GB of memory (test data are scored in 50,000-row chunks, so their HAI scores depend slightly on chunk size). All other detectors use every training row. The deep baseline is an LSTM autoencoder (one-layer encoder and decoder, 64 hidden units, ≈ 52k–58k parameters) on 30-second windows of raw scaled sensors, trained for 25 epochs with Adam; a row’s score is the reconstruction error of the window ending at that row (Malhotra et al., 2016). All models are fitted on rows labelled normal, with one exception: under the SKAB reference protocol the first 400 rows of each run are used as given, and 2.1% of them are labelled anomalous. Table 1 lists the implementation details.

Threshold rules

A score becomes an alarm when it exceeds a threshold τ. Because the comparison should not depend on one rule, two rules are applied identically to every detector:

- Label-free: τ is the 99th percentile of scores on held-out normal data, so about 1% of normal readings are expected to raise an alarm. No anomaly labels are used.

- Validation-MCC: τ maximises MCC on a labelled validation set that contains anomalies, excluding thresholds that flag more than 99% of rows. This is semi-supervised calibration and is reported as such.

A third rule, maximising validation F1, is used only to show how much the initial analysis’ choice of F1 mattered.

Splits

SKAB, pooled protocol.

As in the initial analysis, each fault run is split chronologically into its first 80% and last 20%. The training portions of 20% of the runs (whole runs, fixed seed) form a labelled validation set; the model is trained on the anomaly-free run plus the normal rows of the remaining training portions; the last 20% of every run is the test set (7,503 rows, 30.0% anomalous).

SKAB, benchmark protocol.

The SKAB reference implementation trains a model per run on its first 400 rows and tests on the rest. This protocol is also reported, with the label-free threshold set from each run’s own training scores, so results can be read against the SKAB leaderboard.

SWaT, standard protocol.

Detectors are trained on the normal run from row 21,601 to 85% of its length; the last 15% is held-out normal data for the label-free threshold; the first third of the attack period is the labelled validation set for the validation-MCC threshold. Threshold-free metrics and label-free results are reported on the full attack period; both threshold rules are also compared on its last two thirds, which neither rule has seen.

HAI.

The same design is used: training on the three normal files except the last 15% of the third, which is held-out normal data for the label-free threshold; the first test file is the labelled validation set for the validation-MCC threshold; metrics are reported on all five test files and, for the rule comparison, on test files 2–5. Bootstrap blocks are one-hour blocks within each test file.

Metrics and uncertainty

AUROC and average precision (AUPRC) summarise ranking quality independent of any threshold. At a threshold, precision, recall, F1, MCC, false-alarm rate on normal rows, event recall (the fraction of contiguous anomalous segments with at least one alarm) and median detection delay are reported. MCC is

MCC=TP⋅TN−FP⋅FN(TP+FP)(TP+FN)(TN+FP)(TN+FN)

which is zero for any predictor independent of the labels, including one that flags every row; F1 for that predictor equals 2π/(1+π) for anomaly prevalence π (0.46 on the SKAB test set), which is why F1 alone can credit a detector that separates nothing (Chicco & Jurman, 2020). Precision is bounded by prevalence in the same way: with false-alarm rate f and recall r, precision=πr/(πr+(1−π)f), so the same detector looks much worse in precision on SWaT (π=0.121) than on a test set built with π=0.315, as in the initial analysis. Test rows are strongly autocorrelated, so 95% confidence intervals are obtained by resampling whole blocks with replacement: the 34 test files on SKAB and non-overlapping one-hour blocks on SWaT. The same resamples are used for every detector, so paired differences are valid. Isolation Forest is repeated over ten seeds and the LSTM autoencoder over three seeds on SKAB.

Drift and retraining

The retraining study simulates deployment on SWaT. An initial model is fitted on the first 50,000 normal rows after start-up (about 14 hours). The remainder of the normal week and then the attack period are streamed in one-hour batches; each batch is scored before it can be used for retraining. Four policies are compared: static (never retrain); drift-triggered (retrain when the mean two-sample Kolmogorov–Smirnov statistic between the batch and the current reference, D=1dj=1dsupxFjref(x)−Fjwin(x) (Massey, 1951), exceeds the 95th percentile of its within-initial-window values); periodic (the same number of retrains as the triggered policy, evenly spaced); and always (after every batch). Each retrain uses the last five batches, with rows selected in one of three ways: oracle (ground-truth normal rows, as in the initial analysis), predicted-normal (rows the current model did not flag) or all rows. After every fit the threshold is the 99th percentile of the fit-window scores. Isolation Forest and Elliptic Envelope are tested. The scaler is fitted once on the initial window. The KS statistic is used instead of its p-value because, with thousands of rows per batch, p-values vanish for negligible differences, whereas D is a bounded effect size.

Resource measurement

Each detector is fitted and timed in its own freshly started (spawned) process inside a Docker container run with --cpus=1 --memory=1g on an ARM64 (Apple M4) host, with BLAS threads limited to one; the container’s control group confirmed a 1.0-CPU quota and a 1,024 MB memory limit. Reported are fit time, serialised model size, load time from the serialised form, single-reading latency over 1,000 calls after one warm-up call (median, 95th and 99th percentiles), batch throughput on 10,000 rows, and memory in two parts: the process’s resident memory after the interpreter, libraries and data are loaded (baseline), and the peak increase above that baseline while fitting and scoring (model memory, from the kernel’s high-water mark reset after loading). The container-wide peak and memory events are read from the control group. The cost of recomputing the rolling features for one new reading from a 30-row buffer is reported separately. This is a capped container, not an edge device: a physical device’s slower cores, thermal limits and I/O are not represented, so latencies are lower bounds.

Results

SKAB under the pooled protocol

Table 2 and Fig. 1 (left) report SKAB under the pooled protocol. Every detector is close to chance. The highest AUROC is Isolation Forest’s 0.575 (95% CI 0.504–0.645; mean 0.575 ± 0.018 over ten seeds), and its CI barely excludes 0.5; the CIs of the top five detectors overlap, and the paired difference between the two best (Isolation Forest and COPOD) is 0.012 (CI −0.066 to 0.092). The LSTM autoencoder is below chance (AUROC 0.442, three seeds within ± 0.001). With a calibrated threshold the best MCC is 0.113 (COPOD; CI 0.045–0.190). The label-free rule gives MCC within ± 0.05 of zero for every detector: thresholds learned from normal data in this pooled setting do not separate faults.

SWaT under the standard protocol

Table 3 and Fig. 1 (right) report SWaT. All ten detectors rank attack rows well (AUROC 0.825–0.858) and their CIs overlap almost completely; the difference between the top two (ECOD and One-Class SVM) is 0.005 (CI −0.033 to 0.044). Isolation Forest’s AUROC over ten seeds is 0.824 ± 0.008. The LSTM autoencoder ranks attacks as well as the classical detectors (AUROC 0.852, AUPRC 0.763, the highest AUPRC).

The detectors separate once a threshold is applied. With the label-free rule, which targets 1% false alarms on held-out normal-week data, the z-score baseline reaches MCC 0.716 (CI 0.589–0.819), One-Class SVM 0.660 and Isolation Forest 0.637, with false-alarm rates of 1.4–3.8% on the attack period’s normal rows and alarms in 63–86% of the 35 attack segments. LOF, PCA reconstruction and the LSTM autoencoder instead raise alarms on 68–73% of the attack period’s normal rows, although their threshold gave exactly 1% on held-out normal-week data: their scores shift between the normal week and the attack week independently of the attacks, so their thresholds do not transfer. Elliptic Envelope shows the opposite failure, with very few alarms (MCC 0.008). When a labelled third of the attack period is used to calibrate thresholds, MCC on the remaining two thirds rises for most detectors (One-Class SVM 0.828, z-score 0.827, COPOD 0.802), but not for the LSTM autoencoder (0.266), LOF (0.246) or PCA reconstruction (0.143).

HAI

Table 4 reports HAI. Unlike SWaT, the detectors do separate. PCA reconstruction (AUROC 0.929, CI 0.899–0.954), LOF (0.920) and the LSTM autoencoder (0.916) lead, followed by One-Class SVM and the z-score (0.895); the difference between the top two is 0.008 (CI −0.009 to 0.025). HBOS (0.728) and Elliptic Envelope (0.663, CI 0.608–0.716) are clearly worse, and Isolation Forest sits in between (0.837; 0.815 ± 0.009 over ten seeds). Label-free thresholds transfer well on HAI: held-out normal data from the training period predicted the test-period false-alarm rate closely (0.4–1.6% for every classical detector against a 1% target; 9.1% for the LSTM autoencoder), and the leading five detectors raised alarms in all 50 attacks. With these thresholds MCC is 0.597–0.627 for PCA reconstruction, LOF, One-Class SVM and the z-score. Calibrating on a labelled test file raises MCC on the remaining files only slightly (by 0.05–0.06) for the leaders, but by 0.33 for the LSTM autoencoder, whose label-free threshold was too low.

Across the two plants, the detectors that lead differ: PCA reconstruction and LOF, which failed under label-free thresholds on SWaT, lead on HAI, while Elliptic Envelope trails on HAI. Only the z-score and One-Class SVM are in the dependable group on both (label-free MCC 0.716 and 0.660 on SWaT, 0.597 and 0.627 on HAI).

How much protocol choices change the conclusions

Split definition (SWaT).

Scoring the same detectors with the same code on the re-upload’s layout (the split of the initial analysis: train on the normal rows between attacks, test on 31.5% attacks) gives AUROC 0.840–0.934 and reorders the detectors: HBOS ranks first there and last under the standard split (Fig. 2, left). Under that layout the training data come from the same five days as the test attacks, so the normal-week to attack-week shift described above never appears in evaluation.

Threshold rule (SWaT).

The initial analysis tuned one contamination value on Isolation Forest and applied it to every model. On the same layout, per-model validation-MCC thresholds (with the revised detector settings of Section 4) raise the z-score’s MCC from 0.180 to 0.844, PCA reconstruction’s from 0.002 to 0.834 and LOF’s from 0.105 to 0.830, while Elliptic Envelope, the initial winner at 0.766, moves only to 0.810. The initial ranking therefore reflected mainly the shared threshold, not the detectors.

Training protocol (SKAB).

Under the SKAB reference protocol (one model per run, trained on its first 400 rows), the same detectors are far better than under pooled training: mean per-run AUROC reaches 0.794 (PCA reconstruction), 0.785 (LOF) and 0.771 (One-Class SVM), against 0.489–0.575 pooled (Fig. 2, right; Table 5). The ranking inverts: Isolation Forest, the best pooled detector, is among the weakest per run, and ECOD and COPOD fall to chance because, with only 400 training rows and a 99th-percentile threshold, they almost never alarm. Of the first 400 rows, 2.1% are labelled anomalous, so this protocol trains on slightly contaminated data, as in the reference implementation. Absolute numbers are not directly comparable with the SKAB leaderboard (best F1 0.78 at a 13.6% false-alarm rate for a convolutional autoencoder), because features, thresholds and post-processing differ; detectors here reach similar F1 (0.72–0.73) only at much higher false-alarm rates (47–71%), while Elliptic Envelope offers a more balanced operating point (F1 0.644, false alarms 20.2%, MCC 0.361).

Threshold rule (SKAB).

On the pooled SKAB test set, Isolation Forest’s MCC is 0.005 with the label-free rule, 0.097 with validation-MCC and 0.118 with validation-F1; the F1 rule buys its MCC with recall 0.71 at higher false-alarm rates. Differences of this size are within the CIs.

Feature choices.

Removing the elapsed-time feature and computing features before dropping fault rows changed SKAB AUROC by at most 0.02 (Isolation Forest 0.591 initial pipeline, 0.555 with elapsed time, 0.575 revised), well within the CIs; these corrections matter for validity, not for the numbers.

Drift and Retraining

Does normal operation drift?

Streaming SWaT’s genuine normal week through a static Isolation Forest fitted on its first 14 hours, the share of normal readings that raise an alarm, after a transient spike in the first day, stays near the design rate of 1% for about two days and then jumps, at roughly hour 48, to 40–94% for the rest of the week (mean 12% over the first 24 streamed hours, 76% over the last 24; Fig. 3, left). The change is a step, consistent with a change in operating regime, rather than a slow trend. The Elliptic Envelope fitted on the same window does not show this: its false-alarm rate over the normal week is 0.9%. Drift is therefore real, but whether it matters depends on the detector. Isolation Forest operates on all 120 raw-scale features, whereas the Elliptic Envelope operates on ten whitened principal components; a plausible, untested explanation is that slow level changes in individual sensors fall mostly outside those components.

Retraining budget.

A retrain attempt is a batch on which the policy fires; it becomes a refit only if the selected five-hour window holds more than 1,000 rows, otherwise the current model is kept. To compare policies at equal cost, the periodic policy was run every P∈{1,2,3,4,6,12,24,48} batches, which traces how detection skill grows with the number of refits; the KS-triggered policy is then read against that curve (Table 6, Fig. 3).

With labelled retraining data.

When retraining rows are selected with ground-truth labels (as in the initial analysis), Isolation Forest’s skill over the attack period rises steadily with the number of refits: MCC 0.167 static (73% of normal rows flagged), 0.276 with 5 refits, 0.380 with 40, 0.456 with 80, 0.485 with 120 and 0.537 when refitting after every batch (239). The KS-triggered policy reached 0.488 with 74 refits (79 attempts), above the periodic curve at 80 refits and level with periodic retraining at 120 refits, so triggering buys a modest saving in the number of refits. For the Elliptic Envelope, no budget changes the outcome: MCC 0.707–0.731 at every periodic budget and 0.709 for the KS policy, against 0.723 for the static model.

Without labels, retraining can fail.

A deployed detector does not know which recent rows are normal. When Isolation Forest retrains on the rows it did not flag, it enters a self-reinforcing failure: once it flags most rows, too few remain to refit on, attempts stop turning into refits (193 attempts gave 52 refits under the KS policy; 61 attempts gave 10 refits at P=4), and the detector ends up flagging everything (MCC 0.000–0.033 at every budget except P=2, 0.286). Retraining on all recent rows avoids that collapse but absorbs the attacks into the model of normal behaviour: recall falls to 0.27–0.51 and MCC to 0.074–0.303 whenever refits are frequent. The Elliptic Envelope tolerates predicted-normal selection at every budget (MCC 0.683–0.748). With all-rows retraining it loses skill only when refits are frequent (MCC 0.332–0.530 with 40 or more refits, 0.683–0.717 with 20 or fewer), because each frequent refit takes in a larger share of the ongoing attacks. The benefit of adaptation reported in the initial analysis therefore depended on labels that a deployment would not have.

Cost.

On the host, an Isolation Forest refit on five hours of data took about 0.08 s and an Elliptic Envelope refit about 1.1 s. The KS check (120 features against a reference of up to 50,000 rows) cost 51–117 s of CPU over the 244 batches in the label-selected and all-rows runs (0.2–0.5 s per batch) and 483 s in the collapsing predicted-normal run. For Isolation Forest the checks alone cost five times more than refitting after every batch (96 s against 19 s, oracle selection), and every-batch refitting gave the best detection, so a drift trigger does not pay. A trigger can pay only when refits are expensive relative to the check, as for the Elliptic Envelope, which in this stream did not need retraining at all.

A note on the Elliptic Envelope’s instability.

In this streaming setting the static Elliptic Envelope, fitted on 14 hours, reaches MCC 0.723, yet in Table 3, fitted on the full normal week with its threshold set on the week’s last 15%, its label-free MCC is 0.008. The two runs differ only in the training window and threshold-calibration data. One consistent reading is that drift within the normal week inflates the end-of-week scores used for calibration, producing a threshold too high to fire. Either way, a detector whose performance swings this much with the calibration window is fragile in deployment, a property a single benchmark number hides.

Resource Cost per Reading

Table 7 and Fig. 4 report the cost of each detector fitted on SWaT-shaped data (120 features; 50,000 normal rows) inside the one-CPU, 1 GB ARM64 container. Single-reading latency spans more than five orders of magnitude. The z-score scores a reading in 0.002 ms; PCA reconstruction, HBOS, Elliptic Envelope and One-Class SVM take 0.05–0.16 ms; LOF 0.89 ms; Isolation Forest 2.3 ms, dominated by per-call overhead across its 100 trees, since its batch throughput is about 850,000 rows/s. ECOD and COPOD, as implemented in PyOD, keep their training data and rescore against it on every call: 698 and 739 ms per reading and 235 MB models. Above a baseline of about 444 MB (Python, libraries and data), the detectors themselves needed 71–228 MB, except ECOD and COPOD (516 and 527 MB). The container’s memory use reached its 1 GB limit only during the ECOD and COPOD runs: the kernel recorded 3,127 reclaim events at the limit and no out-of-memory kills, so the cap was enforced and both completed. On SKAB-shaped data (40 features) the pattern is the same at smaller scale (Isolation Forest 2.3 ms; ECOD and COPOD 118–125 ms, 39 MB models, about 260 MB model memory). The LSTM autoencoder takes 0.78 ms per 30-second window with 58k parameters, but needs a deep-learning runtime (peak process memory 335 MB for inference alone). Recomputing the rolling features for one new reading from a 30-row buffer with the pipeline’s pandas code costs 3.2 ms, more than any detector except ECOD and COPOD, so in a streaming deployment incremental feature updates would matter more than the choice among the fast detectors. These are lower bounds for a physical device. (An earlier measurement in forked child processes over-counted memory, because a forked child’s resident set includes pages shared with its parent; it is not used here.)

Discussion

Table 8 condenses what each protocol choice did to the measured outcome and what it implies for someone selecting or deploying a detector.

Ranking quality is not the bottleneck; thresholds and protocols are.

On SWaT’s standard split, ten detectors that span statistical, distance, density, tree, copula and recurrent families are statistically indistinguishable in AUROC. This agrees with large benchmark studies that find no consistent winner and strong simple baselines (Audibert et al., 2022; Sarfraz et al., 2024). What separates them in use is whether a threshold set on normal data still works later: the z-score, One-Class SVM and Isolation Forest kept false alarms near their design rate on the attack week’s normal data, while LOF, PCA reconstruction and the LSTM autoencoder did not. A detector comparison that reports only threshold-free ranking, or only thresholds tuned on test-period labels, hides exactly the property an operator cares about. HAI shows the other side: there the detectors do differ in ranking ability, the threshold learned from normal data transfers, and a different group leads. Which detector is best is therefore a property of the plant and of the protocol together; what carried across both plants was the robustness of the simplest detectors, the z-score and One-Class SVM.

Why the initial conclusions changed.

Three protocol choices in the initial analysis each favoured a particular answer. Training on the attack period’s own normal rows removed the normal-week to attack-week shift from the evaluation. A contamination value tuned on Isolation Forest and shared by all detectors made detectors with differently scaled scores look poor. Retraining on ground-truth-normal rows made adaptation look free of risk. None of these choices is unusual, and each was documented in the code. Their effect became visible only when the same detectors were rerun under alternative protocols. Studies that compare detectors should therefore report at least one alternative threshold rule and state where their training and test data come from in time.

What adaptation needs.

Normal operation does drift, and some detectors are much more sensitive to it than others. Where retraining helps, it helps only if the retraining data are clean; with label-free selection Isolation Forest either collapsed into flagging everything or learned the attacks as normal. In practice this argues for one of three things: retraining data vetted by an operator; a slow-changing detector (here, the Elliptic Envelope on whitened components), whose static form was as good as any adaptive variant; or adaptation of the threshold rather than the model. For cheap models such as Isolation Forest, a drift test can cost more than the retrain it is meant to save.

Cost.

Single-reading latencies span almost six orders of magnitude, from microseconds (z-score) to about 700 ms (ECOD and COPOD in PyOD on SWaT, which store and rescore against their training data), and model sizes from 1 KB to 235 MB. The batch-amortised figures used in the initial analysis (around 0.002 ms per row for Isolation Forest) understate the real single-reading cost (about 2.3–2.8 ms) by three orders of magnitude, because per-call overhead dominates. For a monitor that scores one reading per second, all detectors except possibly ECOD and COPOD fit comfortably; the deciding constraints are memory and threshold stability, not speed.

Threats to Validity and Limitations

- Dataset copy. SWaT results come from a public re-upload reconstructed to the standard split; seven channels are blank in its normal run, so 24 of the 25 continuous sensors were used and actuators were excluded. The reconstruction reproduces the published row counts, but the results must be confirmed on the official iTrust release before they are cited as SWaT results.

- Process coverage. SKAB and SWaT are water processes; HAI adds turbine and boiler processes but through a hardware-in-the-loop simulator. Rotating machinery under real wear, electrical faults and naturally occurring (rather than induced or scripted) anomalies are not covered; WADI (Ahmed et al., 2017) and run-to-failure machinery data are natural next steps. The drift and cost studies were run on SWaT only.

- Detector configurations. Hyperparameters other than thresholds were fixed at common defaults; One-Class SVM and LOF were trained on a 20,000-row subsample on SWaT and HAI, and ECOD and COPOD on a 100,000-row subsample on HAI; the LSTM autoencoder is small and was run with one seed on SWaT. Larger or tuned deep models were not tested.

- Metrics. Event recall and median delay are reported, but range-aware metrics such as VUS-PR (Paparrizos et al., 2022) were not computed. Bootstrap blocks (files, hours) reduce but do not remove dependence between blocks.

- Drift design. One stream, one window length (five hours), one KS threshold rule and two detectors were studied; ADWIN-style detectors and threshold-only adaptation were not compared. Policies are compared without confidence intervals because the stream is a single realisation.

- Resource measurements. A Docker container capped at one CPU and 1 GB on an Apple M4 host is not a physical edge device: slower cores, thermal limits and real I/O are absent, so latencies are lower bounds. Model memory is the peak increase above a measured baseline and includes transient scoring buffers. Energy was not measured.

- SKAB protocols. The SKAB reference protocol trains on rows of which 2.1% are labelled anomalous, and its label-free threshold is set from each run’s own training scores; results are not directly comparable with the SKAB leaderboard, whose features, thresholds and post-processing differ.

- Search scope. The literature search used a general web search and publisher pages rather than database-level systematic queries; statements that a comparison was not found are limited to that search.

Conclusion

Evaluating ten lightweight and deep detectors on SKAB, SWaT and HAI under documented, standard protocols answered the four research questions as follows. (RQ1) On SWaT’s standard split the detectors rank attacks equally well (AUROC 0.83–0.86, overlapping intervals); on HAI they separate, with PCA reconstruction and LOF leading (0.93, 0.92) and Elliptic Envelope trailing (0.66); on SKAB with pooled training, none is reliably better than chance. Only the z-score and One-Class SVM were dependable under label-free thresholds on both SWaT and HAI. (RQ2) Protocol choices change the conclusions more than detector choice does: the SWaT split definition reorders the detectors, a shared contamination value had made the z-score look useless (MCC 0.18 against 0.84 with its own threshold), and SKAB’s per-run protocol inverts the pooled ranking. Under thresholds set only from normal data, the simple z-score, One-Class SVM and Isolation Forest were the most dependable on SWaT (MCC 0.64–0.72). (RQ3) Normal operation drifts enough to break a static Isolation Forest; retraining restores it only when the retraining data are known to be normal, drift-triggered retraining saved some refits but cost more in drift checks than it saved, and label-free retraining failed. (RQ4) Single-reading costs range from microseconds to about 700 ms and model sizes from kilobytes to hundreds of megabytes under a one-CPU, 1 GB cap; the per-reading feature update (3.2 ms) costs more than most detectors.

The initial analysis reported dataset-specific winners, a large benefit from drift adaptation, and sub-millisecond inference; none of these survived re-examination. What did survive is a clearer and more useful message for practitioners: choose the detector for its threshold stability and retraining needs rather than its benchmark rank, and report the protocol in enough detail that a reader can tell which of these the numbers measure. Next steps are confirmation on the official SWaT release, a third dataset from a different machine class, threshold-only adaptation, and measurement on physical edge hardware.

Data and Code Availability

Code, configuration and all result files are available in the project repository (experiments/revision/, results/revised/). SKAB is public (Katser & Kozitsin, 2020). SWaT must be requested from iTrust, SUTD; the re-upload used and the reconstruction of the standard split are documented in the code. Software used: scikit-learn (Pedregosa et al., 2011), PyOD (Zhao et al., 2019) and PyTorch.

Disclosure of Interests

The authors have no competing interests to declare.

References

Ahmad, S., Lavin, A., Purdy, S., & Agha, Z. (2017). Unsupervised real-time anomaly detection for streaming data. Neurocomputing, 262, 134–147. doi:10.1016/j.neucom.2017.04.070

Ahmed, C. M., Palleti, V. R., & Mathur, A. P. (2017). WADI: A water distribution testbed for research in the design of secure cyber physical systems. In Proc. 3rd Int. Workshop on Cyber-Physical Systems for Smart Water Networks (CySWater) (pp. 25–28). doi:10.1145/3055366.3055375

Audibert, J., Michiardi, P., Guyard, F., Marti, S., & Zuluaga, M. A. (2020). USAD: Unsupervised anomaly detection on multivariate time series. In Proc. 26th ACM SIGKDD Int. Conf. on Knowledge Discovery & Data Mining. doi:10.1145/3394486.3403392

Audibert, J., Michiardi, P., Guyard, F., Marti, S., & Zuluaga, M. A. (2022). Do deep neural networks contribute to multivariate time series anomaly detection? Pattern Recognition, 132, 108945. doi:10.1016/j.patcog.2022.108945

Banbury, C., et al. (2021). MLPerf Tiny benchmark. In Proc. NeurIPS Datasets and Benchmarks Track. arXiv:2106.07597

Bifet, A., & Gavaldà, R. (2007). Learning from time-changing data with adaptive windowing. In Proc. SIAM Int. Conf. on Data Mining (pp. 443–448). doi:10.1137/1.9781611972771.42

Breunig, M. M., Kriegel, H.-P., Ng, R. T., & Sander, J. (2000). LOF: Identifying density-based local outliers. In Proc. ACM SIGMOD Int. Conf. on Management of Data (pp. 93–104). doi:10.1145/342009.335388

Chandola, V., Banerjee, A., & Kumar, V. (2009). Anomaly detection: A survey. ACM Computing Surveys, 41(3), 1–58. doi:10.1145/1541880.1541882

Chicco, D., & Jurman, G. (2020). The advantages of the Matthews correlation coefficient (MCC) over F1 score and accuracy in binary classification evaluation. BMC Genomics, 21, 6. doi:10.1186/s12864-019-6413-7

Dasari, S. (2026). When to retrain: An empirical study of retraining policies for streaming ML under concept drift, budget, and latency constraints. arXiv preprint arXiv:2608.19488.

Deng, A., & Hooi, B. (2021). Graph neural network-based anomaly detection in multivariate time series. In Proc. AAAI Conf. on Artificial Intelligence, 35(5), 4027–4035. doi:10.1609/aaai.v35i5.16523

Gama, J., Žliobaitė, I., Bifet, A., Pechenizkiy, M., & Bouchachia, A. (2014). A survey on concept drift adaptation. ACM Computing Surveys, 46(4), 1–37. doi:10.1145/2523813

Garg, A., Zhang, W., Samaran, J., Savitha, R., & Foo, C.-S. (2021). An evaluation of anomaly detection and diagnosis in multivariate time series. IEEE Transactions on Neural Networks and Learning Systems. doi:10.1109/TNNLS.2021.3105827

Goh, J., Adepu, S., Junejo, K. N., & Mathur, A. (2016). A dataset to support research in the design of secure water treatment systems. In Proc. 11th Int. Conf. on Critical Information Infrastructures Security (CRITIS) (pp. 88–99). doi:10.1007/978-3-319-71368-7_8

Goldstein, M., & Uchida, S. (2016). A comparative evaluation of unsupervised anomaly detection algorithms for multivariate data. PLoS ONE, 11(4), e0152173. doi:10.1371/journal.pone.0152173

Han, S., Hu, X., Huang, H., Jiang, M., & Zhao, Y. (2022). ADBench: Anomaly detection benchmark. In Advances in Neural Information Processing Systems 35 (Datasets and Benchmarks Track). arXiv:2206.09426

Hundman, K., Constantinou, V., Laporte, C., Colwell, I., & Soderstrom, T. (2018). Detecting spacecraft anomalies using LSTMs and nonparametric dynamic thresholding. In Proc. 24th ACM SIGKDD Int. Conf. on Knowledge Discovery & Data Mining. doi:10.1145/3219819.3219845

Katser, I. D., & Kozitsin, V. O. (2020). Skoltech Anomaly Benchmark (SKAB) [Data set]. Kaggle. doi:10.34740/KAGGLE/DSV/1693952

Kim, S., Choi, K., Choi, H.-S., Lee, B., & Yoon, S. (2022). Towards a rigorous evaluation of time-series anomaly detection. In Proc. AAAI Conf. on Artificial Intelligence, 36(7), 7194–7201. doi:10.1609/aaai.v36i7.20680

Lee, W.-J. (2017). Anomaly detection and severity prediction of air leakage in train braking pipes. International Journal of Prognostics and Health Management, 8(3). doi:10.36001/ijphm.2017.v8i3.2662

Li, D., Chen, D., Jin, B., Shi, L., Goh, J., & Ng, S.-K. (2019). MAD-GAN: Multivariate anomaly detection for time series data with generative adversarial networks. In Proc. Int. Conf. on Artificial Neural Networks (ICANN). arXiv:1809.04758

Li, Z., Zhao, Y., Hu, X., Botta, N., Ionescu, C., & Chen, G. H. (2023). ECOD: Unsupervised outlier detection using empirical cumulative distribution functions. IEEE Transactions on Knowledge and Data Engineering, 35(12), 12181–12193. doi:10.1109/TKDE.2022.3159580

Liu, Q., & Paparrizos, J. (2024). The elephant in the room: Towards a reliable time-series anomaly detection benchmark. In Advances in Neural Information Processing Systems 37 (Datasets and Benchmarks Track) (pp. 108231–108261).

Liu, F. T., Ting, K. M., & Zhou, Z.-H. (2008). Isolation forest. In Proc. 8th IEEE Int. Conf. on Data Mining (pp. 413–422). doi:10.1109/ICDM.2008.17

Lu, J., Liu, A., Dong, F., Gu, F., Gama, J., & Zhang, G. (2019). Learning under concept drift: A review. IEEE Transactions on Knowledge and Data Engineering, 31(12), 2346–2363. doi:10.1109/TKDE.2018.2876857

Malhotra, P., Ramakrishnan, A., Anand, G., Vig, L., Agarwal, P., & Shroff, G. (2016). LSTM-based encoder-decoder for multi-sensor anomaly detection. ICML Anomaly Detection Workshop. arXiv:1607.00148

Massey, F. J. (1951). The Kolmogorov-Smirnov test for goodness of fit. Journal of the American Statistical Association, 46(253), 68–78. doi:10.1080/01621459.1951.10500769

Mathur, A. P., & Tippenhauer, N. O. (2016). SWaT: A water treatment testbed for research and training on ICS security. In Proc. Int. Workshop on Cyber-physical Systems for Smart Water Networks (CySWater) (pp. 31–36). doi:10.1109/CySWater.2016.7469060

Mukherjee, R. (2021). A modified energy statistic for unsupervised anomaly detection. International Journal of Prognostics and Health Management, 12(1). doi:10.36001/ijphm.2021.v12i1.1323

Paparrizos, J., Boniol, P., Palpanas, T., Tsay, R. S., Elmore, A., & Franklin, M. J. (2022). Volume under the surface: A new accuracy evaluation measure for time-series anomaly detection. Proceedings of the VLDB Endowment, 15(11), 2774–2787. doi:10.14778/3551793.3551830

Pedregosa, F., et al. (2011). Scikit-learn: Machine learning in Python. Journal of Machine Learning Research, 12, 2825–2830.

Poenaru-Olaru, L., Karpova, N., Cruz, L., Rellermeyer, J. S., & van Deursen, A. (2024). Is your anomaly detector ready for change? Adapting AIOps solutions to the real world. In Proc. IEEE/ACM 3rd Int. Conf. on AI Engineering – Software Engineering for AI (CAIN) (pp. 222–233). doi:10.1145/3644815.3644961

Rabanser, S., Günnemann, S., & Lipton, Z. (2019). Failing loudly: An empirical study of methods for detecting dataset shift. In Advances in Neural Information Processing Systems 32. arXiv:1810.11953

Rousseeuw, P. J., & Van Driessen, K. (1999). A fast algorithm for the minimum covariance determinant estimator. Technometrics, 41(3), 212–223. doi:10.1080/00401706.1999.10485670

Sarfraz, M. S., Chen, M.-Y., Layer, L., Peng, K., & Koulakis, M. (2024). Position: Quo vadis, unsupervised time series anomaly detection? In Proc. 41st Int. Conf. on Machine Learning, PMLR 235, 43461–43476.

Schmidl, S., Wenig, P., & Papenbrock, T. (2022). Anomaly detection in time series: A comprehensive evaluation. Proceedings of the VLDB Endowment, 15(9), 1779–1797. doi:10.14778/3538598.3538602

Schölkopf, B., Platt, J. C., Shawe-Taylor, J., Smola, A. J., & Williamson, R. C. (2001). Estimating the support of a high-dimensional distribution. Neural Computation, 13(7), 1443–1471. doi:10.1162/089976601750264965

Shin, H.-K., Lee, W., Yun, J.-H., & Kim, H. (2020). HAI 1.0: HIL-based augmented ICS security dataset. In Proc. 13th USENIX Workshop on Cyber Security Experimentation and Test (CSET).

Shin, H.-K., Lee, W., Yun, J.-H., & Min, B.-G. (2021). Two ICS security datasets and anomaly detection contest on the HIL-based augmented ICS testbed. In Proc. Cyber Security Experimentation and Test Workshop (CSET ’21).

Shyu, M.-L., Chen, S.-C., Sarinnapakorn, K., & Chang, L. (2003). A novel anomaly detection scheme based on principal component classifier. In Proc. IEEE ICDM Workshop on Foundations and New Directions of Data Mining (pp. 172–179).

Tatbul, N., Lee, T. J., Zdonik, S., Alam, M., & Gottschlich, J. (2018). Precision and recall for time series. In Advances in Neural Information Processing Systems 31. arXiv:1803.03639

Tuli, S., Casale, G., & Jennings, N. R. (2022). TranAD: Deep transformer networks for anomaly detection in multivariate time series data. Proceedings of the VLDB Endowment, 15(6), 1201–1214. doi:10.14778/3514061.3514067

Weiss, B., & Brundage, M. (2021). Measurement and evaluation for prognostics and health management (PHM) for manufacturing operations: Summary of an interactive workshop highlighting PHM trends. International Journal of Prognostics and Health Management, 12(1). doi:10.36001/ijphm.2021.v12i1.2653

Wu, R., & Keogh, E. (2023). Current time series anomaly detection benchmarks are flawed and are creating the illusion of progress. IEEE Transactions on Knowledge and Data Engineering, 35(3), 2421–2429. doi:10.1109/TKDE.2021.3112126

Zhao, Y., Nasrullah, Z., & Li, Z. (2019). PyOD: A Python toolbox for scalable outlier detection. Journal of Machine Learning Research, 20(96), 1–7.

| Detector | Configuration | Score (higher = more anomalous) |
| --- | --- | --- |
| z-score | training mean and s.d. per feature | maxj¦xj−μj¦/σj |
| PCA-recon | components for 95% of variance | squared reconstruction error |
| Elliptic Envelope | 10 whitened PCs; robust MCD (default support) | Mahalanobis distance |
| Isolation Forest | 100 trees, default subsample, seeds 0–9 | −score_samples |
| One-Class SVM | RBF, ν=0.1, gamma = scale | −decision_function |
| LOF | k=20, novelty mode | −score_samples |
| ECOD, COPOD, HBOS | PyOD defaults | decision_function |
| LSTM-AE | 1-layer LSTM enc./dec., 64 units, window 30, Adam, lr 10−3, | window MSE |
|  | batch 256, 25 epochs, stride 2 (4 on HAI); raw sensors, own scaler |  |
| Software: Python 3.14 (host) / 3.12 (container), NumPy 2.4.6, SciPy 1.17.1, pandas 3.0.3, |  |  |
| scikit-learn 1.9.0, PyOD 3.6.7, PyTorch 2.12.1. Seeds: 0 unless stated; bootstrap: NumPy default_rng(0). |  |  |

Implementation details. All classical detectors receive the same standardised feature matrix (StandardScaler fitted on training rows only); scores are oriented so that higher means more anomalous.

| Detector | AUROC [95% CI] | AUPRC | MCC LF | MCC VM [95% CI] | Event rec. VM |  |
| --- | --- | --- | --- | --- | --- | --- |
| Isolation Forest | 0.575 [0.504, 0.645] | 0.338 | 0.005 | 0.097 [−0.026, 0.233] | 0.94 |  |
| COPOD | 0.558 [0.507, 0.621] | 0.320 | −0.042 | 0.113 [0.045, 0.190] | 1.00 |  |
| ECOD | 0.535 [0.476, 0.601] | 0.315 | −0.016 | 0.066 [−0.010, 0.159] | 1.00 |  |
| Elliptic Envelope | 0.520 [0.459, 0.581] | 0.305 | −0.050 | 0.051 [−0.079, 0.182] | 0.45 |  |
| z-score | 0.513 [0.477, 0.554] | 0.307 | −0.051 | −0.032 [−0.102, 0.039] | 0.48 |  |
| HBOS | 0.510 [0.420, 0.602] | 0.315 | 0.041 | 0.012 [−0.116, 0.152] | 0.85 |  |
| One-Class SVM | 0.498 [0.463, 0.533] | 0.300 | −0.035 | −0.042 [−0.106, 0.009] | 0.73 |  |
| PCA-recon | 0.489 [0.455, 0.517] | 0.294 | −0.046 | −0.047 [−0.104, −0.005] | 0.76 |  |
| LOF | 0.489 [0.450, 0.533] | 0.299 | 0.007 | −0.021 [−0.073, 0.032] | 0.82 |  |
| LSTM-AE | 0.442 [0.392, 0.500] | 0.267 | −0.075 | −0.078 [−0.165, −0.007] | 0.27 |  |

SKAB, pooled protocol (test: last 20% of each of 34 runs, 7,503 rows, 30.0% anomalous). AUROC and MCC with 95% block-bootstrap CIs over files. MCC columns use the label-free (LF) or validation-MCC (VM) threshold; event recall uses VM; 33 fault events.

AUROC with 95% block-bootstrap confidence intervals (blocks: test files on SKAB, one-hour blocks on SWaT and HAI). Dashed line: chance. On SKAB (pooled) no detector is clearly above chance; on SWaT (standard split) all ten are well above chance and statistically indistinguishable; on HAI a leading group separates from HBOS and Elliptic Envelope.

| Detector | AUROC [95% CI] | AUPRC | MCC LF [95% CI] | FAR LF | Event rec. LF | MCC VM |
| --- | --- | --- | --- | --- | --- | --- |
| ECOD | 0.858 [0.779, 0.925] | 0.703 | 0.310 [0.196, 0.395] | 0.001 | 0.20 | 0.688 |
| One-Class SVM | 0.853 [0.772, 0.916] | 0.752 | 0.660 [0.506, 0.776] | 0.038 | 0.86 | 0.828 |
| COPOD | 0.852 [0.772, 0.920] | 0.728 | 0.584 [0.438, 0.683] | 0.001 | 0.17 | 0.802 |
| LSTM-AE | 0.852 [0.755, 0.917] | 0.763 | 0.134 [0.059, 0.209] | 0.726 | 0.89 | 0.266 |
| LOF | 0.852 [0.759, 0.916] | 0.744 | 0.183 [0.129, 0.250] | 0.683 | 1.00 | 0.246 |
| z-score | 0.847 [0.769, 0.907] | 0.738 | 0.716 [0.589, 0.819] | 0.014 | 0.86 | 0.827 |
| PCA-recon | 0.845 [0.756, 0.910] | 0.747 | 0.144 [0.073, 0.214] | 0.704 | 0.91 | 0.143 |
| Isolation Forest | 0.838 [0.747, 0.912] | 0.720 | 0.637 [0.480, 0.755] | 0.034 | 0.63 | 0.738 |
| Elliptic Envelope | 0.829 [0.737, 0.899] | 0.528 | 0.008 [−0.029, 0.056] | 0.014 | 0.74 | 0.643 |
| HBOS | 0.825 [0.734, 0.903] | 0.554 | 0.500 [0.333, 0.629] | 0.088 | 0.63 | 0.443 |

SWaT, standard protocol (test: full attack period, 449,919 rows, 12.1% attacks, 35 attack segments). AUROC and MCC with 95% block-bootstrap CIs (one-hour blocks). LF: label-free threshold (99th percentile of held-out normal-week scores); FAR: false-alarm rate on the attack period’s normal rows; VM: validation-MCC threshold calibrated on the first third, scored on the last two thirds.

| Detector | AUROC [95% CI] | AUPRC | MCC LF [95% CI] | FAR LF | Event rec. LF | MCC VM |
| --- | --- | --- | --- | --- | --- | --- |
| PCA-recon | 0.929 [0.899, 0.954] | 0.641 | 0.624 [0.572, 0.670] | 0.016 | 1.00 | 0.678 |
| LOF | 0.920 [0.895, 0.948] | 0.617 | 0.626 [0.570, 0.675] | 0.016 | 1.00 | 0.691 |
| LSTM-AE | 0.916 [0.887, 0.948] | 0.585 | 0.336 [0.284, 0.391] | 0.091 | 1.00 | 0.664 |
| One-Class SVM | 0.895 [0.858, 0.930] | 0.590 | 0.627 [0.573, 0.677] | 0.012 | 1.00 | 0.662 |
| z-score | 0.895 [0.860, 0.928] | 0.539 | 0.597 [0.544, 0.648] | 0.013 | 1.00 | 0.653 |
| ECOD | 0.865 [0.817, 0.906] | 0.470 | 0.519 [0.446, 0.580] | 0.007 | 0.78 | 0.515 |
| COPOD | 0.853 [0.795, 0.900] | 0.479 | 0.531 [0.467, 0.591] | 0.004 | 0.80 | 0.513 |
| Isolation Forest | 0.837 [0.783, 0.884] | 0.359 | 0.414 [0.343, 0.484] | 0.004 | 0.66 | 0.425 |
| HBOS | 0.728 [0.652, 0.792] | 0.115 | 0.145 [0.078, 0.214] | 0.011 | 0.42 | 0.121 |
| Elliptic Envelope | 0.663 [0.608, 0.716] | 0.114 | 0.179 [0.119, 0.248] | 0.007 | 0.48 | 0.177 |

HAI 21.03 (test: five files, 402,005 rows, 2.2% attacks, 50 attacks). AUROC and label-free MCC with 95% block-bootstrap CIs (one-hour blocks). FAR: false-alarm rate on normal test rows; VM: validation-MCC threshold calibrated on test file 1, scored on files 2–5.

Same detectors, same code, different protocol. Left: SWaT AUROC under the re-upload layout used in the initial analysis versus the standard split. Right: SKAB AUROC with one model trained on all runs (pooled) versus one model per run trained on its first 400 rows (SKAB reference protocol; mean over runs).

| Detector | F1 | FAR | MAR | MCC | Mean run AUROC |
| --- | --- | --- | --- | --- | --- |
| Elliptic Envelope | 0.644 | 0.202 | 0.445 | 0.361 | 0.749 |
| z-score | 0.722 | 0.468 | 0.213 | 0.331 | 0.766 |
| LOF | 0.723 | 0.556 | 0.168 | 0.302 | 0.785 |
| One-Class SVM | 0.732 | 0.712 | 0.077 | 0.277 | 0.771 |
| PCA-recon | 0.731 | 0.707 | 0.081 | 0.276 | 0.794 |
| HBOS | 0.603 | 0.306 | 0.457 | 0.239 | 0.674 |
| Isolation Forest | 0.470 | 0.201 | 0.641 | 0.174 | 0.664 |
| COPOD | 0.044 | 0.024 | 0.977 | −0.004 | 0.548 |
| ECOD | 0.027 | 0.022 | 0.986 | −0.029 | 0.493 |

SKAB reference protocol (per run: first 400 rows train, remainder test; label-free threshold from each run’s training scores). FAR and MAR as defined by the SKAB benchmark (false-alarm and missed-alarm rates).

|  | Selection | P=1 | P=2 | P=3 | P=6 | P=12 | P=24 | P=48 | KS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IF | oracle | 0.537 (239) | 0.485 (120) | 0.456 (80) | 0.380 (40) | 0.325 (20) | 0.254 (10) | 0.276 (5) | 0.488 (74) |
|  | pred.-normal | 0.001 (52) | 0.286 (105) | 0.004 (25) | 0.000 (7) | 0.000 (4) | 0.033 (4) | 0.005 (1) | 0.025 (52) |
|  | all rows | 0.226 (244) | 0.147 (122) | 0.303 (81) | 0.124 (40) | 0.301 (20) | 0.225 (10) | 0.200 (5) | 0.188 (79) |
| EE | oracle | 0.711 (239) | 0.707 (120) | 0.717 (80) | 0.726 (40) | 0.719 (20) | 0.724 (10) | 0.731 (5) | 0.709 (74) |
|  | pred.-normal | 0.683 (238) | 0.729 (120) | 0.734 (80) | 0.711 (39) | 0.738 (20) | 0.748 (10) | 0.740 (5) | 0.725 (73) |
|  | all rows | 0.335 (244) | 0.405 (122) | 0.418 (81) | 0.423 (40) | 0.717 (20) | 0.701 (10) | 0.683 (5) | 0.332 (79) |

Retraining budget on SWaT: MCC over the attack period (point-wise) for each detector and retraining-data selection, at every periodic budget P (refit attempt every P hourly batches; 244 batches in total) and for the KS-triggered policy. The number in brackets is the number of successful refits. Static: Isolation Forest 0.167, Elliptic Envelope 0.723.

Left: false-alarm rate of a static Isolation Forest over SWaT’s genuine normal week (threshold set to flag 1% of its training window). Middle and right: detection skill (MCC) over the attack period against the number of successful refits; lines are periodic retraining at different budgets, stars the KS-triggered policy, the dashed line the static model. Colours give the retraining-data selection.

| Detector | Fit (s) | Load (ms) | p50 (ms) | p95 (ms) | Batch rows/s | Size (KB) | Model mem. (MB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| z-score | 0.006 | 0.12 | 0.002 | 0.002 | 7,188,644 | 2 | 107 |
| HBOS | 1.10 | 0.24 | 0.050 | 0.053 | 812,430 | 802 | 164 |
| PCA-recon | 0.019 | 0.18 | 0.058 | 0.063 | 1,242,616 | 52 | 71 |
| Elliptic Envelope | 4.22 | 0.31 | 0.151 | 0.161 | 5,866,393 | 503 | 89 |
| One-Class SVM | 3.10 | 2.78 | 0.158 | 0.172 | 10,049 | 1,942 | 228 |
| LOF | 2.55 | 1.88 | 0.892 | 0.934 | 7,745 | 22,188 | 128 |
| Isolation Forest | 0.072 | 7.46 | 2.255 | 2.353 | 853,367 | 1,149 | 85 |
| ECOD | 0.83 | 196.5 | 698.2 | 956.7 | 8,960 | 235,157 | 516 |
| COPOD | 1.11 | 222.6 | 739.3 | 833.9 | 11,203 | 235,157 | 527 |

Cost per detector in a Docker container capped at one CPU and 1 GB RAM (linux/arm64 on an Apple M4 host; single BLAS thread; one spawned process per detector). SWaT-shaped input, 120 features, 50,000 training rows. Latency: one reading per call, 1,000 calls after warm-up. Model memory: peak resident-memory increase above the 444 MB baseline during fitting and scoring.

Single-reading latency against serialised model size (both log scale) in the capped container, for SKAB-shaped (40 features) and SWaT-shaped (120 features) input.

| Choice | Evidence in this study | Implication |
| --- | --- | --- |
| Training/test split | SWaT re-upload layout vs standard split: AUROC 0.84–0.93 vs 0.83–0.86; HBOS first vs last. SKAB pooled vs per-run: chance-level vs run AUROC up to 0.79, ranking inverted | State the time span and source of training and test data; use a benchmark’s reference protocol |
| Threshold rule | Shared contamination vs per-model calibration: z-score MCC 0.18 vs 0.84. Label-free vs label-calibrated on SWaT: LOF, PCA, LSTM-AE MCC 0.13–0.18 vs 0.14–0.27, false alarms 68–73% | Report a label-free threshold as the deployable result; treat label-calibrated results as an optimistic reference |
| Retraining data | Oracle labels: Isolation Forest MCC 0.17→0.54. Predicted-normal: collapse to flag-everything. All rows: attacks absorbed, MCC ≤0.30 | Do not claim adaptation gains from label-selected retraining; vet retraining data or use slowly changing detectors |
| Retraining trigger | KS trigger: 0.488 with 74 refits vs periodic 0.456 (80), 0.485 (120); KS checks 96 s vs every-batch refits 19 s | Compare triggers against a budget curve, including the cost of the check |
| Cost measurement | Batch-averaged vs single-reading latency for Isolation Forest: 0.0019 vs 2.3 ms; feature update 3.2 ms | Report single-reading latency and memory above a stated baseline |

Summary: how each evaluation choice changed the measured outcome in this study, and the practical implication.